# United Business Services
Multi-Branch Network Setup (Cisco Packet Tracer)

STEP 1: DEVICES ADD KARO
- Cisco Packet Tracer open karo
- Add karo:
  * 3 Routers (2811 ya 1941)
  * 3 Switches (2960)
  * Har switch ke sath 2 PCs
- Routers rename karo:
  * HO-Router
  * Branch115-Router
  * Branch65-Router

STEP 2: CONNECTIONS
- PC to Switch: Copper Straight-Through
- Switch to Router (G0/0): Copper Straight-Through
- Router to Router: Serial DCE cable

STEP 3: IP ADDRESSING
Head Office:
- Network: 192.168.79.0/24
- Gateway: 192.168.79.1

Branch 1:
- Network: 192.168.115.0/24
- Gateway: 192.168.115.1

Branch 2:
- Network: 192.168.65.0/24
- Gateway: 192.168.65.1

STEP 4: ROUTER CONFIGURATION

Head Office Router:
enable
conf t
hostname HO-Router
int g0/0
ip address 192.168.79.1 255.255.255.0
no shut
int s0/0/0
ip address 10.0.0.1 255.255.255.252
clock rate 64000
no shut
int s0/0/1
ip address 10.0.0.5 255.255.255.252
clock rate 64000
no shut
ip route 192.168.115.0 255.255.255.0 10.0.0.2
ip route 192.168.65.0 255.255.255.0 10.0.0.6
end

Branch115 Router:
enable
conf t
hostname Branch115-Router
int g0/0
ip address 192.168.115.1 255.255.255.0
no shut
int s0/0/0
ip address 10.0.0.2 255.255.255.252
no shut
ip route 0.0.0.0 0.0.0.0 10.0.0.1
end

Branch65 Router:
enable
conf t
hostname Branch65-Router
int g0/0
ip address 192.168.65.1 255.255.255.0
no shut
int s0/0/1
ip address 10.0.0.6 255.255.255.252
no shut
ip route 0.0.0.0 0.0.0.0 10.0.0.5
end

STEP 5: PC CONFIGURATION
- IP manually set karo
Example:
Main Office PC:
IP: 192.168.79.10
Subnet: 255.255.255.0
Gateway: 192.168.79.1

STEP 6: TESTING
- Ping karo PC se PC
- Agar reply aaye, network working hai

# DHCP Configuration (Automatic IP Assignment)

STEP 7: DHCP CONFIGURATION ON ROUTERS

Head Office Router (192.168.79.1 network):
enable
conf t
ip dhcp pool HO_Pool
network 192.168.79.0 255.255.255.0
default-router 192.168.79.1
dns-server 8.8.8.8
exit

Branch115 Router (192.168.115.1 network):
enable
conf t
ip dhcp pool BR115_Pool
network 192.168.115.0 255.255.255.0
default-router 192.168.115.1
dns-server 8.8.8.8
exit

Branch65 Router (192.168.65.1 network):
enable
conf t
ip dhcp pool BR65_Pool
network 192.168.65.0 255.255.255.0
default-router 192.168.65.1
dns-server 8.8.8.8
exit

STEP 8: PC CONFIGURATION FOR DHCP
- PC IP setting: Automatic (DHCP)
- PC will receive IP, subnet, gateway and DNS from respective router DHCP pool

STEP 9: TESTING DHCP
- Ping from any PC to another PC or router
- Check assigned IP by typing 'ipconfig' on PC (Windows simulation in Packet Tracer)